A.)   301 and 303 would need to programmed with the primary and back up receiver numbers.

B.)    310 and 311 would need to be programmed with the account last 4.

C.)    310 should be programmed as account last 4 * 66 as if you don’t, it may not save…. If you have more than one partition, each partition has an account number. (i.e. partition 2 account last for is programmed in section 312 and so on.

D.)   350 is where you would program the format. Programming 0300 in this section should be sufficient…

E.)    351, 359, 367, 375, and 376 are your communicator call directions. The is usually toggle sections but it depends on if you have the ADT defaults loaded and whether or not you have an alpha/fixed keypad as far as how you program these sections…. So only phone 1 is needed here, if further clarification of these sections are needed, you would need to call in….

F.)    380 is another toggle section the same as 351 and etc… You only need option 1 and option 5 if you have a secondary receiver number.

G.)   Make sure 7 is off in section 381 but it is yet another toggle section just like 380 so what you see and you programming depend on the aforementioned… However, I would make sure it is programmed for Auto CID reporting as if you don’t you will have to program specific report codes for each signal, (i.e if zone 1 is a burg, you would have to program it in 320, first to digits would be 3a or 31 and etc… depending on the type of burg if you want to be this granular).

H.)   The report code section beginning in 320 depend heavily on the programming of 381 but if you take the advice of programming the panel for Auto CID reporting, all report code values for whatever signal that needs to report should be something other than 00.